How to use your GPS Unit (Garmin eTrex 20)

Basic Setup

Back button lets you go back to the previous screen.

Insert 2 AA batteries. To insert batteries, hold on to the silver D-ring on the back of the unit. Turn it, and pull off the back panel. At the end of each lesson, be sure to remove the batteries.

- Turn it on.

Up and down 

buttons are used to zoom in and out OR to toggle up and down.

Thumb Stick toggles up and down. Pushing on it makes a selection. It is also used to adjust brightness level.

Menu

Power and Light button

Press and hold to turn on and off.

Keep the Backlight off whenever possible because it uses up the battery quickly. Battery life is 25 hours

Select a Profile

Profiles are a collection of settings that optimize your device based on how you are using it.

- Select Profile Change.

- Select a profile. (typical choices in a school setting are “Geocaching” or “Recreational”)

* The GPS unit usually can pick up satellite transmissions only when you are outside.

* When using eTrex20s outdoors with students, always bring extra AA batteries.

* To save battery power, always make sure that students turn off all the units at the end of each lesson.

You may want to assign a student to make sure all units are turned off.

General Tips

* The eTrex20 compass only works when you’re moving.

How to connect the Garmin eTrex20 to a computer

- Connect the USB cable to a USV port on your computer.

- Pull up the weather cap from the mini-USB port.

- Plug the small end of the USB cable into the mini-USB port.

Your device and memory card appear as “mounted volumes” on Macs (or “removable drives” in My Computer on Windows).

Resetting the Device

Press Menu

Select Reset

Select Reset All Setting

Select Yes

If the device stops responding

- Remove the batteries.

- Reinstall the batteries.

NOTE: This does not erase any of your data or settings.

To delete all waypoints

Press Menu

Select Waypoint Manager

Select Menu again

Select Delete All

To delete all tracks

Press Menu

Select Track Manager

Select Menu again

Select Delete All

Waypoints, Routes, and Tracks on Garmin eTrex20

Waypoints – are locations your record and store in the device.

Creating a Waypoint – saves your current location as a waypoint

1. Select Mark Waypoint.

2. Select Done

Finding a Waypoint

1. Select Where To? > Waypoint

2. Select a Waypoint

Editing a Waypoint –before you edit a waypoint, you must create a waypoint

1. Select Waypoint Manager

2. Select a Waypoint

3. Select an item to edit

4. Enter the new information

5. Select Done

Deleting a Waypoint

1. Select Waypoint Manager

2. Select a waypoint

3. Select menu > Delete > Yes

Routes – a sequence of waypoints that leads you to your final destination

Creating a Route

1. Select Route Planner > Create Route > Select First Point.

2. Select a category.

3. Select the first point in the route.

4. Select Use > Select Next Point.

5. Repeat steps 2-4 until the route is complete.

6. Select Back to save the route.

Editing the Name of a Route

1. Select Route Planner.

2. Select a route.

3. Select Change Name.

4. Enter the new name.

5. Select Done.

Editing a Route

1. Select Route Planner.

2. Select a route

3. Select Edit Route.

4. Select a point.

5. Select an option.

* To view the point on the map, select Review.

* To change the order of the point on the route, select Move Down or Move Up.

* To add an additional point on the route, select Insert.

6. Select back to save the route.

Viewing a Route on the Map

1. Select Route Planner.

2. Select a route.

3. Select View Map.

Deleting a Route

1. Select Route Planner.

2. Select a route.

3. Select Delete Route.

Viewing the Active Route

1. While navigating a route, select Active Route.

2. Select a point in the route to view additional details.

Reversing a Route

1. Select Route Planner.

2. Select a route.

3. Select Reverse Route.

Tracks – a recording of your path. The track log contains information about points along the recorded path, including time, location, and elevation for each point.

Recording Track Logs

1. Select Setup > Tracks > Track Log.

2. Select Record, Do Not Show, or Record, Show on Map (a line on the map indicates your track.

3. Select Record Method.

4. Select an option:

* To record tracks at a variable rate that creates an optimum representation of your tracks, select Auto.

* To record tracks at a specified distance, select Distance.

* To record tracks at a specified time, select Time.

5. Select Record Interval.

6. Complete an action:

- Select an option to record tracks more or less often. NOTE: Using the Most Often interval provides the most track detail, but fills up the device memory quicker.

- Enter a time or distance, and select Done.

As you move with the device turned on, a track log is created.

Calculating the Size of an Area

1. Select Area Caluculation > Start

2. Walk around the perimeter of the area you want to calculate

3. Select Calculate when finished.

The full Garmin Manual is in the back of this binder.