# Preliminary Fluid Inclusion Micro-Thermometry Study of Quartz Veins from the Virginia Horn Greenstone Terrain

# Prepared by:	Lakehead University Mineralogy and Experimental 			Laboratory (LUMINX)

Prepared for:	Mineral Potential Section,

Division of Lands and Minerals
			Minnesota Department of Natural Resources

# Attn: Dennis Martin, Manager

# Jul 20, 2007

# Scope

This report summaarises the preliminary results for fluid inclusion analysis conducted as part of State of Minnesota, Department of Natural Resources’ Virginia Horn Fluid Inclusion Project.  Included in this report is the overall micro-thermometric data,  calculated maximum fluid salinities and pressure, histograms showing variation of measured temperatures amongst the inclusions analyzed, bivariate plots showing variation among different lithologies, mineralized against unminerralized samples and final comparison with existing equivalent Northern Ontario gold deposits.

# Disclaimer

# Results are representative only of material submitted for examination.  Data and interpretations provided in this report are only preliminary and have not been fully substantiated.

This report has been reviewed and authorized by Dr. Andrew Conly, LUMINX Director.

LUMINX Analytical Staff:

Parisa Sattari (MSc)

Figure 1.  Frequency diagram showing  the data for homogenization and melting temperatures of ice for provided samples.   TH oC varies from low to high temperatures with bulk of it around 150.  Freezing temperature also show high numbers of low melting temperature corresponding to less CO2 bearing and high CO2 bearing  FIAs.

Figure 2. A. Bivariate plot illustrating the distribution of homogenization TH and final melting temperature of ice (Tm ice ) data.  The overall data with out any discrimination is shown in A.

Figure 2. B.   Bivariate plot displaying the same data buy differentiating among different lithological units (ie. Tuff, greywacke, metabasalt, quartz feldspar pophyry, dacite porphyry and Rhyolite)

A

B

A

B

Figure 3.  A.  Bivariate plot showing the distribution pattern of TH & Tmice among mineralized and unmineralized samples.   The pattern in unmineralized samples is equally varying from low to high where as mineralized samples show different pattern.

Figure 3.  B. Bivariate plot showing the distribution pattern of TH and Tm ice in mineralized samples. Tight bimodal clusters are represented by samples of dacite origin where as mineralized Tuff show tight cluster at high temperature (blue triangle).

Figure 4.  Bivariate plot of homogenizations against final melting temperature of ice showing comparison of this data to equivalent gold deposits in Northern Ontario. The illustrated data above are compiled collection of micro-thermometric data from both published literature and our recent studies of similar liquid-vapour fluid inclusion assemblages obtained in quartz veins.  As shown above and Fig 3 b, this bipolar distribution shows similarity to Hemlo, McInTyre and Pamour deposits.

Current observation of FIAS of L-V:

1.  Lithology

TH varies from low temperature to 200oC in tuff samples

Greywacke show a scattered TH around 150 oC

Metabasic rock show lower TH centre (150 oC)

Quartz feldspar porphyry show even scattering through low and high ranges

Dacite porphyry show bimodal pattern  with temperatures in two high and low clusters

Rhyolitic samples show a low temperature cluster

2.  Mineralization

Only mineralized samples show bimodal distribution, non mineralized show a general trend of even scatter ranging from 80 to 340 oC.

Dacite samples show definitive bimodal  pattern among the mineralized rocks.

3. Salinites and pressure

Salinities are calculated using FLINCOR (Brown, P. E. 1989) and as per low TH and presence of CO2, maximum salinities are calculated and range from 2 to 21 % saline solution (Table 1) and low estimated pressure of 57 bar.

Low Tm ice, Teu and presence of CO2 are consistent  with presence of average to high saline aqueous solution containing a mixture of mono and divalent cations of Na/K/Ca/Mg/Fe and different proportions of CO2.

Bimodal salinity is also observed for mineralized samples.

Higher TH corresponds to higher saline brines of more Ca/Mg/K/Na rich (in decreasing order): 8-18 wt% saline with more scatter around higher range

Low TH showing low saline fluids of more Na/k/Ca/Mg: 5-19 wt% saline with high scatter around lower range.

Comparative Studies

Dacite porphyry show close similarity to McInTyre, Pamour and Hemlo type deposits where both salinity and CO2 variations may affect gold mobilization.  They are similar in sense that there are primary or psedo-secondary fluid inclusions of aqueous and CO2 bearing fluids. Low Tmice , Teu and TmCO2  suggest the complex nature of fluids (Na/K/Mg/ca+/- CO2).  Presence of saline brine and CO2 clearly shows heterogeneous entrapment of fluid at time of mineralization but uncertain sources of fluid.

Further study of the mineralized samples and lithologies, dept and their chemistry will assist us in determining variation of K/Na/Ca/Mg contents and CO2  in order to establish the mechanism of gold mobilization via source of fluid.

References

Chi Guoxiang, Dube Benoit and Williamson Kenneth 2002, Preliminary fluid inclusion microthermometry study of fluid evolutions and temperature-pressure conditions in GoldCorp High–grade zone, Red Lake Mine, Ontario, GSC currency research 2002-C27, p. 1-12.

Fleet Michael E., Pan Yuanming, 1991, Cal-Silicate alterations in the Hemlo Gold Deposit, Ontario: Mineral Assemblages, P-T-X Constraints, and significance, Economic Geology, Vol. 87, p. 1104-1120.

Smith Ted J., Cloke Paul L., and Kesler Stephen, 1984, Geochemistry of fluid inclusions from the McInTyre-Hollinger Gold Deposit, Timmins, Ontario, Canada, Economic Geology, Vol. 79, p. 1265-1285.

Walsh James F., and Kesler Stephen, 1988, Fluid inclusion geochemistry of high-grade gold ore at the Pamour Mine, Porcupine Camp, Ontario, Economic Geology, Vol. 83, p. 1347-1367.